- Non-destructive testing by Dye Penetrant Inspection (“DIP”) method for 4 eyes pad and change the color code for 4 pad eyes

- Remove all existing hardware. Be sure to keep all washers, bolt and regular nuts.

- Nylon Safety Sling. Unscrew upper center post from top frame

- Replace Rubber Feet. Grease bolts and add the washer, bolt, and lock nut to medal backing plate and rubber foot

- Prepping for Grab lines. Completely screw top pole back into top frame. Place top assembly into the bottom frame.

- Attaching Grab Lines to Top & Bottom Frames

- Attaching Grab Lines to Bottom Frame. Tip the X-904 on its side and insert inner 8 grab lines to the bottom frame

- Add Center Rubber Foot

- Tensioning the Grab lines

- Add Bottom Bumper. Reuse existing 6”foam

- Add Top Bumper. Replace existing 4”foam

- Add Carrgo and Stretcher Netting

- Stencil on “footing”,”cargo only”

- Add Galvanized Cables

- Add Stabilizer to Sling

- Center Pole Cover & Lanyards

- Place yellow inspection sticker and cover on center pole